NUST SCHOOL OF MECHANICAL & MANUFACTURING ENGINEERING

H-12 SECTOR, ISLAMABAD

EC-803, Computer Vision

Semester Project Viva

M2N2 Model Fusion: EfficientNet-Lite0 + Tiny Recursive Model (TRM)

Course Instructor

Dr. Saqib Nazir

Student Name: Muhammad Qasim Jalil

Student ID: 539433

Section: RAI-2025 | Date: 09 – 12 – 2025

Contents

Questions & Answers for Hybrid CV Architecture Project	2

Question 1: EfficientNetLite0 Channel Calculation	2

Question 2: EfficientNetLite0 CIFAR Adaptation	2

Question 3: MBConvBlock without SE	3

Question 4: TRMBlock Recursive Processing	3

Question 5: TRMNet Internal Logic	4

Question 6: Nano-TRM Adaptation	4

Question 7: HybridEfficientNetTRM Data Flow	5

Question 8: HybridEfficientNetTRM Feature Extractor Initialization	6

Question 9: Modular Design Benefit in Hybrid Model	6

Question 10: SLERP vs. LERP for Weights	8

Question 11: M2N2 split_idx Influence	9

Question 12: SLERP Numerical Stability Debugging	10

Question 13: CosineAnnealingLR Role	11

Question 14: Reproducibility in Training	12

Question 15: Debugging Empty DataLoader	12

Question 16: TRM Input Projection	13

Question 17: Concatenation in TRM	13

Question 18: Output Projection Role	13

Question 19: EfficientNet Stem Convolution	14

Question 20: MBConv Expansion Ratio	14

Question 21: Residual Connection Condition	15

Question 22: Kaiming Initialization	15

Question 23: Adaptive Average Pooling	16

Question 24: M2N2 Merging Strategy	16

Question 25: Checkpointing logic	17

Question 26: Evaluation Function Abstraction	17

Question 27: Optimizer and Scheduler Pairing	18

Question 28: Hybrid Model Initialization	18

Question 29: M2N2 Deep Copy	19

Question 30: EfficientNet Feature Flattening	19

# Questions & Answers for Hybrid CV Architecture Project

This document contains 30 questions and answers covering the project’s codebase, focusing on conceptual understanding, code tracing, debugging, and design decisions. Each question includes a detailed answer, a GitHub URL, and a relevant code snippet.

## Question 1: EfficientNetLite0 Channel Calculation

Question: Why is the _make_divisible function used in EfficientNetLite0’s channel calculation, and what problem does it aim to solve?

Answer: The _make_divisible function ensures that the number of channels in convolutional layers is a multiple of a specified divisor (typically 8). This is done to optimize model performance on certain hardware (e.g., GPUs, TPUs) which can process tensors more efficiently when their dimensions are multiples of specific values. It helps prevent performance bottlenecks and ensures better hardware utilization.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

def _make_divisible(v, divisor=8, min_value=None):
    """
    Ensure that all layers have a channel number divisible by 8.
    This is to optimize for certain hardware.
    """
    if min_value is None:
        min_value = divisor
    new_v = max(min_value, int(v + divisor / 2) // divisor * divisor)
    # Make sure that round down does not go down by more than 10%.
    if new_v < 0.9 * v:
        new_v += divisor
    return new_v

## Question 2: EfficientNetLite0 CIFAR Adaptation

Question: In EfficientNetLite0, what is the purpose of setting the initial convolution’s stride to 1 (instead of the typical 2 for ImageNet) for CIFAR-10 input?

Answer: The original EfficientNet models are designed for larger input images (e.g., 224x224 for ImageNet), where an initial stride of 2 helps quickly reduce spatial dimensions. For CIFAR-10 images (32x32), applying a stride of 2 too early would severely reduce the spatial resolution (to 16x16) too quickly, potentially losing fine-grained features critical for classification. By setting the stride to 1, the model retains the 32x32 resolution at the initial stage, allowing for better feature extraction from the small images.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

# Stem
        in_channels = 3 # RGB input
        out_channels = _make_divisible(32 * width_mult)
        # CIFAR Adaptation: Initial stride 1 to keep 32x32 resolution
        self.stem = ConvBNReLU(in_channels, out_channels, 3, 1, 1)

## Question 3: MBConvBlock without SE

Question: The MBConvBlock in EfficientNetLite0 does not include a Squeeze-and-Excitation (SE) block. What is the rationale behind this design choice for “Lite” versions of EfficientNet?

Answer: Squeeze-and-Excitation (SE) blocks are typically used in EfficientNet models to improve channel-wise feature recalibration, boosting accuracy. However, SE blocks add computational overhead (FLOPs) and increase the number of parameters. “Lite” versions of EfficientNet (like Lite0) are specifically designed for resource-constrained environments (e.g., mobile devices) where minimizing latency and model size is paramount. Removing the SE block is a trade-off that reduces computational cost and parameter count, making the model faster and lighter, often with a minimal drop in accuracy for certain applications.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

class MBConvBlock(nn.Module):
    """
    Mobile Inverted Bottleneck Convolution Block.
    EfficientNet-Lite uses ReLU6 and no Squeeze-and-Excitation (SE).
    """

## Question 4: TRMBlock Recursive Processing

Question: Explain the core idea behind the “recursive processing” in the TRMBlock as implemented in src/models/trm.py.

Answer: The TRMBlock implements a simplified Tiny Recursive Model. The core idea of “recursive processing” is to iteratively refine a latent feature representation (z_latent) by repeatedly passing it, along with the original input features (x_input), through a small neural network (self.trm_net). Each iteration allows the model to process and integrate information multiple times, enabling a form of “reasoning” or deeper analysis over the input features, without significantly increasing the model’s parameter count or depth in a traditional feed-forward sense. This enhances the representational quality of the features before final classification.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py

Code Snippet:

# Initialize latent state z with a projection of the input features
        z_latent = self.input_projection(x_input)

        # Recursive processing
        # In TRM.pdf, Figure 1, the arrow "Update z given x, y, z"
        # and "recursive reasoning" block
        for _ in range(self.num_recursions):
            # Concatenate x_input with current z_latent to form the input to TRMNet
            combined_input_latent = torch.cat((x_input, z_latent), dim=-1)
            z_latent = self.trm_net(combined_input_latent)

## Question 5: TRMNet Internal Logic

Question: In TRMNet (within src/models/trm.py), what is the purpose and effect of the line return self.relu(self.fc1(combined_input_latent)) + self.fc2(self.relu(self.fc1(combined_input_latent)))?

Answer: This line represents the forward pass of the small neural network that performs the actual latent reasoning within each recursion step. It effectively implements a two-layer MLP (fc1 -> relu -> fc2). The addition (+ self.fc2(...)) creates a skip-connection or a residual-like path within this small network. This helps to stabilize training and allows for better gradient flow during the recursive updates of the z_latent state, especially important in deep or recurrent architectures, preventing vanishing/exploding gradients.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py

Code Snippet:

return self.relu(self.fc1(combined_input_latent)) + self.fc2(self.relu(self.fc1(combined_input_latent)))

## Question 6: Nano-TRM Adaptation

Question: The TRMBlock constructor uses hidden_dim=128 and num_recursions=2 by default. How do these parameters reflect the “Nano-TRM” adaptation for resource-constrained environments like the MX450 GPU?

Answer: These parameters (hidden_dim and num_recursions) are crucial for controlling the computational complexity and memory footprint of the TRMBlock. The original TRM paper suggests higher values (e.g., hidden_dim of 512, num_recursions of 6 or more). By reducing hidden_dim to 128, the size of the latent state z_latent and the number of parameters in TRMNet are significantly decreased. Similarly, reducing num_recursions to 2 limits the number of sequential operations. This “Nano-TRM” adaptation makes the module lightweight enough to fit within the 2GB VRAM of the MX450 GPU and allows for faster training iterations.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py

Code Snippet:

def __init__(self, input_dim: int, final_feature_dim: int, hidden_dim: int = 128, num_recursions: int = 2):
        """
        Args:
            input_dim (int): Dimension of the input feature vector from the backbone.
            final_feature_dim (int): Dimension of the output feature vector from TRM (to be used by classifier).
            hidden_dim (int): Latent dimension for recursive processing (D in TRM paper, here 'z' latent dim).
            num_recursions (int): Number of recursive steps (n in TRM paper). Default to 2 for efficiency.
                                  The paper often uses n=2 or n=6.
        """

## Question 7: HybridEfficientNetTRM Data Flow

Question: Describe the high-level data flow through the HybridEfficientNetTRM model, from input image to final prediction.

Answer: The input image first passes through the efficientnet_features component, which acts as a feature extractor (stem, MBConv blocks, head_conv, and adaptive average pooling from EfficientNet-Lite0). The resulting flattened feature vector is then fed into the trm_block for recursive refinement. Finally, the refined features from the trm_block are passed through a classifier (a linear layer) to produce the final logits for classification.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/hybrid_model.py

Code Snippet:

def forward(self, x):
        # Extract features from EfficientNet
        efficientnet_output = self.efficientnet_features(x)
        
        # Refine features using TRM
        trm_output_features = self.trm_block(efficientnet_output)
        
        # Classify the refined features
        logits = self.classifier(trm_output_features)
        return logits

## Question 8: HybridEfficientNetTRM Feature Extractor Initialization

Question: In HybridEfficientNetTRM, why is efficientnet_feature_extractor = EfficientNetLite0(num_classes=1000) initialized with num_classes=1000 when the actual problem is CIFAR-10 (10 classes)?

Answer: The EfficientNetLite0 model is instantiated here specifically to serve as a feature extractor, meaning only its convolutional layers are needed, and its original classification head is discarded. The num_classes=1000 argument provided during its instantiation for efficientnet_feature_extractor is a placeholder. It creates a nn.Linear(1280, 1000) layer internally within the EfficientNetLite0 constructor, but this specific layer is then ignored by the nn.Sequential block which explicitly selects only the stem, blocks, head_conv, avgpool, and Flatten layers, effectively cutting off the original classifier and allowing the TRMBlock and a new classifier to be attached.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/hybrid_model.py

Code Snippet:

# We need to remove its original classifier to attach TRM and a new classifier.
        efficientnet_feature_extractor = EfficientNetLite0(num_classes=1000) # num_classes doesn't matter, we remove head
        self.efficientnet_features = nn.Sequential(
            efficientnet_feature_extractor.stem,
            efficientnet_feature_extractor.blocks,
            efficientnet_feature_extractor.head_conv,
            efficientnet_feature_extractor.avgpool,
            nn.Flatten(1)
        )

## Question 9: Modular Design Benefit in Hybrid Model

Question: The HybridEfficientNetTRM explicitly separates EfficientNetLite0 into efficientnet_features and then attaches the TRMBlock and a new classification head. What is the primary benefit of this modularization when integrating the TRMBlock?

Answer: The primary benefit of this modularization is flexibility and clear separation of concerns. It allows for the EfficientNetLite0 to function purely as a feature extractor, providing a consistent feature representation regardless of the downstream task. The TRMBlock can then be easily inserted to refine these features, and a custom classifier can be added for the specific task (e.g., CIFAR-10). This design makes the model more adaptable, allowing different reasoning modules or classification heads to be swapped in or out without altering the core feature extraction logic.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/hybrid_model.py#L29-L50

Code Snippet:

class HybridEfficientNetTRM(nn.Module):
    """
    Hybrid model combining EfficientNet-Lite0 as a feature extractor
    and TRMBlock for recursive feature refinement, followed by a classifier.
    """
    def __init__(self, num_classes=10, trm_hidden_dim=128, trm_num_recursions=2):
        super(HybridEfficientNetTRM, self).__init__()
        
        # 1. EfficientNet-Lite0 as feature extractor
        # We need to remove its original classifier to attach TRM and a new classifier.
        efficientnet_feature_extractor = EfficientNetLite0(num_classes=1000) # num_classes doesn't matter, we remove head
        self.efficientnet_features = nn.Sequential(
            efficientnet_feature_extractor.stem,
            efficientnet_feature_extractor.blocks,
            efficientnet_feature_extractor.head_conv,
            efficientnet_feature_extractor.avgpool,
            nn.Flatten(1)
        )
        
        # Get the output dimension of EfficientNet's feature extractor
        # This is hardcoded to 1280 for EfficientNet-Lite0 after GAP
        efficientnet_output_dim = 1280 

        # 2. TRM Block for recursive feature refinement
        # TRM will output a refined feature vector
        trm_final_feature_dim = 256 # Define a specific dimension for TRM's output features
        self.trm_block = TRMBlock(
            input_dim=efficientnet_output_dim,
            final_feature_dim=trm_final_feature_dim,
            hidden_dim=trm_hidden_dim,
            num_recursions=trm_num_recursions
        )

        # 3. Final Classifier Head
        self.classifier = nn.Linear(trm_final_feature_dim, num_classes)

## Question 10: SLERP vs. LERP for Weights

Question: The slerp function in src/m2n2_fusion.py is used for interpolating model weights. What is “spherical linear interpolation” and why might it be preferred over simple linear interpolation (LERP) for model weights, especially in the context of model merging?

Answer: Spherical Linear Interpolation (SLERP) interpolates along the shortest arc on a unit n-sphere between two vectors, maintaining a constant speed and curvature. In contrast, Linear Interpolation (LERP) interpolates along a straight line in Euclidean space. For model weights, SLERP is hypothesized to be superior because it might preserve the functional behavior of neural networks better. Weights of a neural network can be thought of as existing in a high-dimensional space, and LERP might pass through “non-functional” regions (e.g., configurations where activation patterns are drastically different), leading to a significant drop in performance. SLERP, by staying on the “surface” of the parameter space, might find a more meaningful path between two functional models, potentially resulting in better merged performance. It is particularly relevant when the “direction” of the weight vectors in high-dimensional space is more important than their absolute magnitude.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py#L4-L21

Code Snippet:

def slerp(t, v0, v1, DOT_THRESHOLD=0.9995):
    """
    Spherical Linear Interpolation for model weights.
    Adapted from: https://github.com/soumith/dcgan.torch/issues/9
    
    Args:
        t (float): Interpolation factor between 0 and 1.
        v0 (torch.Tensor): First tensor.
        v1 (torch.Tensor): Second tensor.
        DOT_THRESHOLD (float): Threshold to switch to linear interpolation if tensors are too close.
                               Prevents numerical instability for very small angles.
    Returns:
        torch.Tensor: Interpolated tensor.
    """
    if not isinstance(v0, torch.Tensor):
        v0 = torch.from_numpy(v0)
    if not isinstance(v1, torch.Tensor):
        v1 = torch.from_numpy(v1)

    dot = torch.sum(v0 * v1)
    
    if dot.item() > DOT_THRESHOLD: # Linear interpolation if too close
        return (1 - t) * v0 + t * v1

    theta_0 = torch.acos(dot.clamp(-1, 1)) # Clamp to avoid NaN from floating point errors
    sin_theta_0 = torch.sin(theta_0)
    
    theta_t = theta_0 * t
    sin_theta_t = torch.sin(theta_t)
    
    s0 = torch.sin(theta_0 - theta_t) / sin_theta_0
    s1 = sin_theta_t / sin_theta_0
    return s0 * v0 + s1 * v1

## Question 11: M2N2 split_idx Influence

Question: In M2N2WeightMerger.merge_state_dicts, explain how the split_idx parameter influences the merging process for model weights. Specifically, what is the effect of having split_idx as None versus a specific integer value?

Answer: The split_idx parameter allows for different mixing ratios to be applied to different parts of the model’s parameters, enabling a “split-point” merging strategy. When split_idx is a specific integer, it divides the ordered list of parameter tensors (from the state_dict) into two parts. For parameters occurring before split_idx, a mix_ratio (e.g., t) is used for interpolation (slerp(t, param_a, param_b)). For parameters at or after split_idx, a complementary mix_ratio (e.g., 1-t) is used (slerp(1-t, param_a, param_b)). This allows for differential merging, where, for instance, early layers might be dominated by one model’s characteristics and later layers by another. If split_idx is None, the mix_ratio is applied uniformly to all parameters (i.e., it behaves as if split_idx is beyond the last parameter), resulting in a global merge without distinct sections.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py

Code Snippet:

if split_idx is None:
            # If no split_idx, apply mix_ratio globally
            split_idx = len(params_a_list) + 1 # Effectively makes all params fall into the first part
        
        # Merge by iterating through the state_dict keys to maintain structure
        param_counter = 0
        for name in merged_state_dict.keys():
            param_a = state_dict_a[name].to('cpu')
            param_b = state_dict_b[name].to('cpu')
            
            # This logic applies split_idx on a per-tensor basis.
            # A more faithful implementation of M2N2's `w_s` would flatten all params,
            # split, then re-pack. For simplicity and managing tensor shapes, we split
            # based on the *number of tensors* encountered.
            
            if param_counter < split_idx:
                # First part of the model (f_wm in Eq. 2)
                merged_state_dict[name] = slerp(mix_ratio, param_a, param_b)
            else:
                # Second part of the model (f_1-wm in Eq. 2)
                # Note: M2N2 paper has f_1-wm(theta_A^>=ws, theta_B^>=ws)
                # This implies mixing with (1-t) on A and t on B for first part,
                # then (t) on A and (1-t) on B for second part.
                merged_state_dict[name] = slerp(1.0 - mix_ratio, param_a, param_b) 
            
            param_counter += 1

## Question 12: SLERP Numerical Stability Debugging

Question: Suppose slerp is called with two identical, non-zero torch.Tensors v0 and v1, and t=0.5. If DOT_THRESHOLD is set too high (e.g., 1.0 - 1e-6) or due to numerical precision, the condition dot.item() > DOT_THRESHOLD (intended for close vectors) is not met. What could be the direct numerical consequences of this in the slerp calculation, particularly for theta_0 and sin_theta_0, and how might it affect the final result compared to the expected v0?

Answer: If v0 and v1 are identical, dot would be torch.sum(v0 * v0) (i.e., ||v0||^2). If v0 is a unit vector, dot would be 1.0. If dot.item() > DOT_THRESHOLD is not met, theta_0 would be calculated as torch.acos(dot.clamp(-1, 1)). If dot is exactly 1.0, theta_0 would be 0. Consequently, sin_theta_0 would also be 0. Division by sin_theta_0 (theta_0 is 0) would lead to a NaN (Not a Number) or Inf (Infinity) result in the s0 and s1 calculations (sin_theta_0 in the denominator), causing the slerp function to return an incorrect, possibly unstable, result (e.g., all NaNs) instead of simply v0 (or v1). The DOT_THRESHOLD is critical for handling such edge cases by falling back to LERP, which is numerically stable when vectors are almost identical.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py#L4-L21

Code Snippet:

def slerp(t, v0, v1, DOT_THRESHOLD=0.9995):
    """
    Spherical Linear Interpolation for model weights.
    Adapted from: https://github.com/soumith/dcgan.torch/issues/9
    
    Args:
        t (float): Interpolation factor between 0 and 1.
        v0 (torch.Tensor): First tensor.
        v1 (torch.Tensor): Second tensor.
        DOT_THRESHOLD (float): Threshold to switch to linear interpolation if tensors are too close.
                               Prevents numerical instability for very small angles.
    Returns:
        torch.Tensor: Interpolated tensor.
    """
    if not isinstance(v0, torch.Tensor):
        v0 = torch.from_numpy(v0)
    if not isinstance(v1, torch.Tensor):
        v1 = torch.from_numpy(v1)

    dot = torch.sum(v0 * v1)
    
    if dot.item() > DOT_THRESHOLD: # Linear interpolation if too close
        return (1 - t) * v0 + t * v1

    theta_0 = torch.acos(dot.clamp(-1, 1)) # Clamp to avoid NaN from floating point errors
    sin_theta_0 = torch.sin(theta_0)
    
    theta_t = theta_0 * t
    sin_theta_t = torch.sin(theta_t)
    
    s0 = torch.sin(theta_0 - theta_t) / sin_theta_0
    s1 = sin_theta_t / sin_theta_0
    return s0 * v0 + s1 * v1

## Question 13: CosineAnnealingLR Role

Question: What is the role of CosineAnnealingLR in the training process implemented in src/train.py, and why is such a learning rate scheduler beneficial?

Answer: CosineAnnealingLR is a learning rate scheduler that gradually decreases the learning rate from an initial maximum value down to a minimum value following a cosine curve. This schedule is applied over a specified number of training epochs (T_max). It is beneficial because it allows the model to take larger steps in the early stages of training (exploring the loss landscape), then gradually reduces the step size as training progresses (fine-tuning weights). This often leads to better convergence to a flatter minimum in the loss landscape, which can improve generalization performance compared to using a fixed learning rate or simpler step-wise decay.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/train.py#L43-L43

Code Snippet:

optimizer = optim.AdamW(model.parameters(), lr=config['learning_rate'], weight_decay=config['weight_decay'])
    scheduler = CosineAnnealingLR(optimizer, T_max=config['epochs'])

## Question 14: Reproducibility in Training

Question: In the train_model function within src/train.py, how is reproducibility ensured when running experiments with different seeds?

Answer: Reproducibility is ensured by the set_seed function, which is called at the beginning of train_model using the config['seed'] value. This function sets the random seed for Python’s random module, numpy, and torch (both CPU and CUDA operations). By fixing these random number generators, the initialization of model weights, the order of data shuffling, and other stochastic operations during training become deterministic, allowing for consistent results when the same seed is used. The override_seed argument allows overriding the config file’s seed from the command line for multi-seed experimental runs.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/train.py

Code Snippet:

# Seed for reproducibility
    set_seed(config['seed'])

## Question 15: Debugging Empty DataLoader

Question: During training, if train_loader is empty or batch_size is too large (leading to an empty last batch in train_loader), what kind of runtime error might running_loss / len(train_loader) produce in src/train.py and how could it be mitigated?

Answer: If train_loader is empty, len(train_loader) would be 0. A division by zero error (ZeroDivisionError) would occur when calculating running_loss / len(train_loader). This typically happens if the dataset is empty, the batch_size is excessively large (larger than the dataset size), or if there’s an issue with data loading that results in no batches being yielded. To mitigate this, a check can be added before the division: train_loss = running_loss / len(train_loader) if len(train_loader) > 0 else 0.0 (or raise a warning/error if an empty loader is unexpected).

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/train.py

Code Snippet:

train_loss = running_loss / len(train_loader)

## Question 16: TRM Input Projection

Question: What is the purpose of the input_projection layer in TRMBlock (src/models/trm.py), and how does its output relate to the hidden_dim parameter?

Answer: The input_projection layer is a linear transformation (nn.Linear) that maps the incoming feature vector from the backbone (with dimension input_dim) to the initial latent state z_latent used for recursive reasoning. The output dimension of this projection is set to hidden_dim. This allows the model to project the high-dimensional input features (e.g., 1280 from EfficientNet) into a lower-dimensional latent space (e.g., 128 for Nano-TRM) suitable for efficient recursive processing.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py#L46-L46

Code Snippet:

# Project input x to an initial latent z
        # In TRM.pdf, initially z is an embedding of x.
        self.input_projection = nn.Linear(input_dim, hidden_dim)

## Question 17: Concatenation in TRM

Question: In the recursive loop of TRMBlock, why are x_input and z_latent concatenated before being passed to trm_net? Why not just pass z_latent?

Answer: Concatenating x_input (the original features) with the current z_latent (the evolving reasoning state) ensures that the original context is always available to the network at every step of the recursion. If only z_latent were passed, the model might “forget” the original input details over multiple iterations, as the latent state evolves. Providing x_input acts as a constant reference or “anchor” to the source data, allowing the reasoning process to stay grounded in the actual input features while refining the latent representation.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py#L65-L66

Code Snippet:

# Concatenate x_input with current z_latent to form the input to TRMNet
            combined_input_latent = torch.cat((x_input, z_latent), dim=-1)
            z_latent = self.trm_net(combined_input_latent)

## Question 18: Output Projection Role

Question: What transformation does self.output_projection perform in TRMBlock, and why is it necessary before the final classification?

Answer: self.output_projection is a linear layer that maps the final refined latent state z_latent (of dimension hidden_dim, e.g., 128) to the desired output feature dimension final_feature_dim (e.g., 256). This step is necessary to decouple the internal reasoning capacity (controlled by hidden_dim) from the feature representation size expected by downstream components (like the classifier). It allows the TRM block to output a standardized feature vector size regardless of its internal configuration.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/trm.py#L52-L54

Code Snippet:

# Output head to project the final latent state to the desired final_feature_dim
        # These features will then be passed to a final classification layer.
        self.output_projection = nn.Linear(hidden_dim, final_feature_dim)

## Question 19: EfficientNet Stem Convolution

Question: In EfficientNetLite0 (src/models/efficientnet_lite.py), the first layer is defined as self.stem = ConvBNReLU(in_channels, out_channels, 3, 1, 1). What does the kernel size of 3 and padding of 1 imply about the spatial dimensions of the output feature map (assuming stride 1)?

Answer: A convolution with kernel size k=3, stride s=1, and padding p=1 preserves the spatial dimensions of the input. The output height/width is calculated as Hout=⌊Hin+2p-ks+1⌋. Substituting the values: Hout=⌊Hin+21-31+1⌋=Hin. Thus, the spatial resolution remains unchanged (32x32 for CIFAR-10) after passing through the stem, which is consistent with the design choice to avoid early downsampling.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

# CIFAR Adaptation: Initial stride 1 to keep 32x32 resolution
        self.stem = ConvBNReLU(in_channels, out_channels, 3, 1, 1)

## Question 20: MBConv Expansion Ratio

Question: In MBConvBlock (src/models/efficientnet_lite.py), what is the role of expand_ratio and how does it affect the hidden_dim?

Answer: The expand_ratio determines how much the channel dimension is expanded in the intermediate phase of the inverted bottleneck block. The hidden_dim is calculated as in_channels * expand_ratio. By expanding the channels (typically by a factor of 6), the depthwise convolution can operate in a higher-dimensional space, potentially capturing richer features, before the 1x1 point-wise convolution projects (“squeezes”) them back down to the out_channels dimension. This “inverted bottleneck” structure (narrow-wide-narrow) is key to MobileNet/EfficientNet efficiency.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

def __init__(self, in_channels, out_channels, kernel_size, stride, expand_ratio):
        super(MBConvBlock, self).__init__()
        self.stride = stride
        self.in_channels = in_channels
        self.out_channels = out_channels
        
        hidden_dim = in_channels * expand_ratio

## Question 21: Residual Connection Condition

Question: Under what conditions does MBConvBlock (src/models/efficientnet_lite.py) apply a residual connection (x + self.conv(x))?

Answer: The residual connection is applied only if self.use_res_connect is true. This condition is met when two criteria are satisfied: 1) The stride is 1 (spatial dimensions match), and 2) The input channels equal the output channels (in_channels == out_channels). If the stride is greater than 1 (downsampling) or the channel counts differ, a direct element-wise addition is not possible due to tensor shape mismatch.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py#L47-L47

Code Snippet:

hidden_dim = in_channels * expand_ratio
        self.use_res_connect = self.stride == 1 and in_channels == out_channels

## Question 22: Kaiming Initialization

Question: Why is nn.init.kaiming_normal_ used to initialize the convolutional weights in EfficientNetLite0 (src/models/efficientnet_lite.py)?

Answer: Kaiming initialization (also known as He initialization) is specifically designed for neural networks that use ReLU (or ReLU-like, such as ReLU6) activation functions. It initializes weights with a variance that accounts for the non-linear activation, preventing the variance of activations from vanishing or exploding as they propagate through deep networks. This helps in training deeper architectures effectively from the start.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py#L103-L103

Code Snippet:

def _initialize_weights(self):
        for m in self.modules():
            if isinstance(m, nn.Conv2d):
                nn.init.kaiming_normal_(m.weight, mode='fan_out')

## Question 23: Adaptive Average Pooling

Question: In EfficientNetLite0, what does self.avgpool = nn.AdaptiveAvgPool2d(1) ensure about the output tensor shape, regardless of the input image size?

Answer: nn.AdaptiveAvgPool2d(1) forces the spatial dimensions (Height, Width) of the output feature map to be exactly 1x1. Regardless of whether the input feature map is 7x7, 4x4, or any other size (which might vary with input image resolution), this layer averages the features across the entire spatial extent, producing a fixed-size output tensor of shape (Batch, Channels, 1, 1). This allows the following fully connected layers (or Flatten) to work with a consistent input size.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/efficientnet_lite.py

Code Snippet:

# Head
        self.head_conv = ConvBNReLU(in_channels, _make_divisible(1280 * width_mult), 1, 1, 0)
        self.avgpool = nn.AdaptiveAvgPool2d(1)

## Question 24: M2N2 Merging Strategy

Question: In M2N2WeightMerger.merge_state_dicts (src/m2n2_fusion.py), how does the code handle parameter tensors that come before the split_idx versus those after?

Answer: The code iterates through the state dictionary parameters sequentially. A counter param_counter tracks the index of the current parameter tensor. If param_counter < split_idx, the new parameter is a blend of param_a and param_b weighted by mix_ratio (favoring model B if ratio > 0.5). If param_counter >= split_idx, the logic switches to use 1.0 - mix_ratio instead. This effectively creates a “crossover” effect where one part of the model is dominated by Model A’s weights (relative to the mix ratio) and the other part by Model B’s weights (inverted ratio).

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py#L76-L84

Code Snippet:

if param_counter < split_idx:
                # First part of the model (f_wm in Eq. 2)
                merged_state_dict[name] = slerp(mix_ratio, param_a, param_b)
            else:
                # Second part of the model (f_1-wm in Eq. 2)
                # Note: M2N2 paper has f_1-wm(theta_A^>=ws, theta_B^>=ws)
                merged_state_dict[name] = slerp(1.0 - mix_ratio, param_a, param_b)

## Question 25: Checkpointing logic

Question: In src/train.py, what condition triggers the saving of a model checkpoint, and why is this condition important?

Answer: The code saves a model checkpoint only if the current epoch’s validation accuracy (val_accuracy) is strictly greater than the best accuracy recorded so far (best_val_accuracy). This ensures that the saved checkpoint (checkpoint_path) always corresponds to the “best” performing version of the model seen during training (early stopping principle), rather than just the final epoch’s model, which might have overfit or degraded in performance.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/train.py#L87-L88

Code Snippet:

# Save best model
        if val_accuracy > best_val_accuracy:
            best_val_accuracy = val_accuracy
            best_epoch = epoch + 1
            checkpoint_path = os.path.join(checkpoint_dir, f"{config['model_name']}_best_seed_{config['seed']}.pth")
            torch.save(model.state_dict(), checkpoint_path)

## Question 26: Evaluation Function Abstraction

Question: In src/m2n2_fusion.py, the find_best_merge method takes an eval_func argument. Why is this design choice (passing a function) beneficial for the merging utility?

Answer: Passing eval_func as an argument makes the M2N2WeightMerger utility model-agnostic and task-agnostic regarding evaluation. The merging logic doesn’t need to know how to evaluate the model (e.g., which dataset to use, what metric to calculate, whether to use GPU). It simply merges the weights, creates a temporary model, and asks the external eval_func “how good is this?”. This allows the same merging code to be reused for different tasks (CIFAR-10, ImageNet) or metrics (Accuracy, F1-score) by simply supplying a different evaluation callback.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py

Code Snippet:

def find_best_merge(self, model_class, models_to_merge, eval_func, num_mix_ratios=5, num_split_points=5):
        """
        A simplified grid search to find the best merge parameters (mix_ratio, split_idx).
        This approximates the evolutionary search aspect of M2N2 for finding optimal merge.
        
        Args:
            ...
            eval_func (callable): A function that takes a model (nn.Module) and returns its validation accuracy.
            Signature: eval_func(model) -> accuracy (float).

## Question 27: Optimizer and Scheduler Pairing

Difficulty: [E] Learning Objective: Identify standard optimizer setups.

Question: Which optimizer is paired with CosineAnnealingLR in src/train.py, and what is the weight_decay parameter used for?

Answer: The AdamW (Adam with Weight Decay) optimizer is used. The weight_decay parameter (set to 0.0001 in configs) implements L2 regularization, which penalizes large weights during training. This helps prevent overfitting, especially in models with many parameters like EfficientNet, by encouraging the model to learn simpler/smoother weight configurations.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/train.py#L42-L43

Code Snippet:

# Loss and optimizer
    criterion = nn.CrossEntropyLoss()
    optimizer = optim.AdamW(model.parameters(), lr=config['learning_rate'], weight_decay=config['weight_decay'])
    scheduler = CosineAnnealingLR(optimizer, T_max=config['epochs'])

## Question 28: Hybrid Model Initialization

Question: If you were to initialize HybridEfficientNetTRM with num_classes=100, which part of the code would need to be updated to ensure the final classifier output dimension matches?

Answer: You would simply need to instantiate the class with the correct argument: model = HybridEfficientNetTRM(num_classes=100). The __init__ method automatically uses the num_classes argument to define the final linear layer: self.classifier = nn.Linear(trm_final_feature_dim, num_classes). No other internal code changes are required because the variable is propagated correctly to the classifier definition.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/hybrid_model.py

Code Snippet:

# 3. Final Classifier Head
        self.classifier = nn.Linear(trm_final_feature_dim, num_classes)

## Question 29: M2N2 Deep Copy

Question: In M2N2WeightMerger.merge_state_dicts, why is merged_state_dict = deepcopy(state_dict_a) used at the start?

Answer: deepcopy is used to create a completely independent copy of state_dict_a. Since state_dicts in PyTorch contain references to tensors, a simple assignment or shallow copy would mean that modifying merged_state_dict would also modify the original state_dict_a (and thus the weights of model_a). deepcopy ensures that the merging process creates a new set of weights without corrupting the source model’s weights, which might be needed for subsequent comparisons or merges.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/m2n2_fusion.py

Code Snippet:

def merge_state_dicts(self, state_dict_a, state_dict_b, mix_ratio, split_idx=None):
        """
        Merges two model state dictionaries using SLERP and a split point.
        ...
        """
        merged_state_dict = deepcopy(state_dict_a) # Start with a copy of A's weights

## Question 30: EfficientNet Feature Flattening

Question: In HybridEfficientNetTRM, what is the purpose of nn.Flatten(1) in the self.efficientnet_features sequence?

Answer: The output of the preceding layer, efficientnet_feature_extractor.avgpool, is a 4D tensor of shape (Batch, Channels, 1, 1). nn.Flatten(1) flattens this tensor starting from dimension 1, resulting in a 2D tensor of shape (Batch, Channels). This transformation is necessary because the subsequent layer (trm_block or a standard linear classifier) expects a 2D input vector (features per sample), not a 4D spatial map.

Evidence (URL): https://github.com/MQasimJalil/M2N2-Model-Fusion/blob/main/src/models/hybrid_model.py#L36-L36

Code Snippet:

self.efficientnet_features = nn.Sequential(
            efficientnet_feature_extractor.stem,
            efficientnet_feature_extractor.blocks,
            efficientnet_feature_extractor.head_conv,
            efficientnet_feature_extractor.avgpool,
            nn.Flatten(1)
        )